Date: 08/08/2025

### Process Loss Summary Report – Swine Feed Production

Company Name:  M/S FARMISM FOODS LLP

Location: GUWAHATI, ASSAM
Reporting Period: 02/08/2025 to 07/08/2025
Prepared by: Sandeep Kapoor
Date of Report: 08/08/2025

- On 02/08/2025

- Total Raw Material Used (MT)  : 5 T

- Total feed Produced (MT)          : 4.984 T

- Total Process Loss (MT)              : 0.016 T

- Process Loss (%)                           : 0.32 %

#### Other Mechanical Data

#### A)

- Pellet Feeder RPM : 692

- Hygenizer RPM : 785

- Pellet Mill (Amp.) : 158 A

- Temperature at Conditioner 1 : 76° C

- Temperature at Conditioner 2 : 71° C

#### B)

- Pellet Feeder RPM : 1156

- Hygenizer RPM : 1018

- Pellet Mill (Amp.) : 168 A

- Temperature at Conditioner 1 : 87° C

- Temperature at Conditioner 2 : 81° C

#### Moisture Report of Raw material

- Maize   : 12.8 %

- Rice      :  12.0 %

- Wheat :  11.0 %

- Soya     :  11.2 %

- On 04/08/2025

- Total Raw Material Used (MT)  : 5 T

- Total feed Produced (MT)          : 4.877 T

- Total Process Loss (MT)              : 0.123 T

- Process Loss (%)                           : 2.46 %

#### Parameters

#### A)

- At Boiler : 5 – 6 Bar

- Water Level : 65 – 70 %

- Before PRV : 4.5 – 5 Bar

- After PRV : 2.9 – 3 Bar

#### B)

- Pellet Feeder RPM : 1111

- Hygenizer RPM : 1013

- Pellet Mill (Amp.) : 190 - 200 A

- Temperature at Conditioner 1 : 83° C

- Temperature at Conditioner 2 : 82° C

#### Second Lot of Batch Processed

- Total Raw Material Used (MT)  : 10 T

- Total feed Produced (MT)          : 9.728 T

- Total Process Loss (MT)              : 0.272 T

- Process Loss (%)                           : 2.72 %

#### Moisture Report

- After Mixing : 11.38 %

- After Conditioning : 12.27 %

- After Bagging : 10.52 %

#### Parameters

#### A)

- At Boiler : 6 – 7 Bar

- Water Level : 60 – 65 %

- Before PRV : 5 – 6 Bar

- After PRV : 3 Bar

#### B)

- Pellet Feeder RPM : 1106

- Hygenizer RPM : 1076

- Pellet Mill (Amp.) : 210 - 215 A

- Temperature at Conditioner 1 : 89° C

- Temperature at Conditioner 2 : 86° C

#### Third Lot Of batch Processed

- Total Raw Material Used (MT)  : 1 T

- Total feed Produced (MT)          : 0.823 T

- Total Process Loss (MT)              : 0.177 T

- Process Loss (%)                           : 17.7 %

#### Moisture Report

- After Conditioning : 13.55 %

- After Pelleting : 11.55

- After Bagging : 11.63 %

#### Parameters

- Pellet Feeder RPM : 860

- Hygenizer RPM : 700 - 750

- Pellet Mill (Amp.) : 190 - 200

- Temperature at Conditioner 1 : 88° C

- Temperature at Conditioner 2 : 84° C

#### Fourth Lot Of batch Processed

- Total Raw Material Used (MT)  : 5 T

- Total feed Produced (MT)          :

- Total Process Loss (MT)              :

- Process Loss (%)                           :

#### Moisture Report

- After Grinding : 11.27 %

- After Conditioning : 12.38 %

- After Pelleting : 11.75 %

- After Bagging : 10.52 %

#### Parameters

#### A)

- At Boiler : 6 – 7 Bar

- Water Level : 60 – 70 %

#### B)

- Pellet Feeder RPM : 1154

- Hygenizer RPM : 1010

- Pellet Mill (Amp.) : 195 – 210 A

- Temperature at Conditioner 1 : 78° C

- Temperature at Conditioner 2 : 77° C

- On 05/08/2025

- Total Raw Material Used (MT)  : 5 T

- Total feed Produced (MT)          : 4.831 T

- Total Process Loss (MT)              : 0.169 T

- Process Loss (%)                           : 3.38 %

#### Parameters

#### A)

- At Boiler : 6 – 6.5 Bar

- Water Level : 60 – 65 %

- Before PRV : 4 – 5 Bar

- After PRV : 1.91 – 2 Bar

#### Moisture Report

- After Conditioning : 12.95 %

#### B)

- At Boiler : 6 – 6.5 Bar

- Water Level : 60 – 65 %

- Before PRV : 5 – 6 Bar

- After PRV : 1.91 – 2.1 Bar

#### Moisture Report

- After Conditioning : 12.71 %

#### C)

- Pellet Feeder RPM : 1040

- Hygenizer RPM : 1000

- Pellet Mill (Amp.) : 200 - 205 A

- Temperature at Conditioner 1 : 81° C

- Temperature at Conditioner 2 : 77° C

#### Second Lot Of Batch Processed

- Total Raw Material Used (MT)  : 4 T

- Total feed Produced (MT)          : 3.863 T

- Total Process Loss (MT)              : 0.137 T

- Process Loss (%)                           : 3.42 %

#### Parameters

#### A)

- Pellet Feeder RPM : 1100

- Hygenizer RPM : 1100

- Pellet Mill (Amp.) : 184 - 195 A

- Temperature at Conditioner 1 : 72° C

- Temperature at Conditioner 2 : 74° C

#### Moisture Report

- After Conditioning : 12.99 %

#### B)

- Pellet Feeder RPM : 900

- Hygenizer RPM : 800

- Pellet Mill (Amp.) : 190 - 204 A

- Temperature at Conditioner 1 : 72° C

- Temperature at Conditioner 2 : 76° C

#### Moisture Report

- After Conditioning : 12.58 %

#### C)

- Pellet Feeder RPM : 860

- Hygenizer RPM : 800

- Pellet Mill (Amp.) : 195 - 215 A

- Temperature at Conditioner 1 : 68° C

- Temperature at Conditioner 2 : 72° C

#### Moisture Report

- After Conditioning : 12.58 %

- On 06/08/2025

- Total Raw Material Used (MT)  : 10 T

- Total feed Produced (MT)          : 9.84647 T

- Total Process Loss (MT)              : 0.153 T

- Process Loss (%)                           : 1.54 %

#### Parameters

#### A)

- At Boiler : 7 Bar

- Water Level : 55 – 60 %

- Before PRV : 5.5 – 6 Bar

- After PRV : 1.8 – 2 Bar

#### Moisture Report

- After Mixing : 10.84 %

- After Pelleting : 12.73 %

- After Cooling : 10.19 %

#### B)

- Pellet Feeder RPM : 1080

- Hygenizer RPM : 1000

- Pellet Mill (Amp.) : 195 - 205 A

- Temperature at Conditioner 1 : 77° C

- Temperature at Conditioner 2 : 75° C

#### Moisture Report

- After Mixing : 10.86 %

- After Pelleting : 12.22 %

#### Second Lot Of Batch Processed

- Total Raw Material Used (MT)  : 5 T

- Total feed Produced (MT)          : 4.962 T

- Total Process Loss (MT)              : 0.38 T

- Process Loss (%)                           : 0.76 %

#### Moisture Report of Raw material

- Maize   : 14.3 %

- Wheat  : 11.8 %

- B. Rice  : 14.5 – 15.9 % / 9.91 – 10.23 %

#### Parameters

#### A)

- At Boiler       : 6 – 6.5 Bar

- Water Level : 60 %

- Before PRV  : 5 – 5.5 Bar

- After PRV     : 2 – 2.2 Bar

- Pellet Feeder RPM : 1054

- Hygenizer RPM : 800

- Pellet Mill (Amp.) : 172 - 186 A

- Temperature at Conditioner 1 : 84° C

- Temperature at Conditioner 2 : 79° C

#### Third Lot of Batch Processed

- Total Raw Material Used (MT)  : 10 T

- Total feed Produced (MT)          : 9.696 T

- Total Process Loss (MT)              : 0.303 T

- Process Loss (%)                           : 3.03 %

#### Moisture Report

- After Conditioning : 12.31 %

- After Cooling : 10.56 %

#### Parameters

#### A)

- At Boiler                                       : 6 – 6.5 Bar

- Water Level                                 : 60 – 65 %

- Before PRV                                  : 5 Bar

- After PRV                                     : 1.8 Bar

- Pellet Feeder RPM                     : 1152

- Hygenizer RPM                           : 1017

- Pellet Mill (Amp.)                       : 189 - 205 A

- Temperature at Conditioner 1 : 86° C

- Temperature at Conditioner 2 : 81° C

#### 5   On 07/08/2025

- Total Raw Material Used (MT)  : 5 T

- Total feed Produced (MT)          : 4.862 T

- Total Process Loss (MT)              : 0.14T

- Process Loss (%)                           : 2.8 %

#### Moisture Report

- After Mixing   : 10.95%

- After Conditioning : 13.05%

- After Bagging : 11.64 %

#### Parameters

#### A)

- At Boiler                                       : 6.5 Bar

- Water Level                                 : 60 %

- Before PRV                                  : 6 Bar

- After PRV                                     : 2 Bar

- Pellet Feeder RPM                     : 1125

- Hygenizer RPM                           : 853

- Pellet Mill (Amp.)                       : 178 A

- Temperature at Conditioner 1 : 80° C

- Temperature at Conditioner 2 : 79° C

#### Second Lot Of Batch Processed

- Total Raw Material Used (MT)  : 20T

- Total feed Produced (MT)          : 19.68 T

- Total Process Loss (MT)              : 0.372 T

- Process Loss (%)                           : 1.86 %

#### Moisture Report

- After Mixing   : 11.45%

- After Conditioning : 13.05%

- After Bagging : 10.74 %

- After Cooling : 11.51 %

#### Parameters

#### A)

- At Boiler                                       : 7 Bar

- Water Level                                 : 60 %

- Before PRV                                  : 5-5.5 Bar

- After PRV                                     : 2-2.2 Bar

- Pellet Feeder RPM                     : 1109

- Hygenizer RPM                           : 800

- Pellet Mill (Amp.)                       : 166 A

- Temperature at Conditioner 1 : 82° C

- Temperature at Conditioner 2 : 77° C

#### Third Lot Of Batch Processed

- Total Raw Material Used (MT)  : 5T

- Total feed Produced (MT)          : 4.951 T

- Total Process Loss (MT)              : 0.049 T

- Process Loss (%)                           : 0.98 %

#### Moisture Report

- After Mixing   : 11.80%

- After Conditioning : 13.21%

- After Cooling : 11.93 %

#### Parameters

#### A)

- At Boiler                                       : 6 Bar

- Water Level                                 : 60 %

- Before PRV                                  : 5 Bar

- After PRV                                     : 2.5 Bar

- Pellet Feeder RPM                     : 1171

- Hygenizer RPM                           : 781

- Pellet Mill (Amp.)                       : 161 A

- Temperature at Conditioner 1 : 84° C

- Temperature at Conditioner 2 : 80° C