CUSTOMER SERVICE REPORT

| REPORT DATE | 30-09-2025 15:30 | REPORT NO | SRN-4836183 |
| --- | --- | --- | --- |
| COMPANY NAME | YTT INDUSTRIES PRIVATE LIMITED | CALL TRACKING NUMBER | CC-01817341 |
| SITE PERSON NAME | Mr Pankaj | TYPE OF CALL | Commissioning |
| SITE PERSON NUMBER | 9793293629 | FABRICATION NUMBER | BWGS034059 |
| SITE PERSON EMAIL ID |  | MODEL | EG 15-10 |
| SITE LOCATION |  | PRODUCT | EPSAC - SEPSAC |
| SALE DEALER |  | PRE COMMISSIONING DATE | 30-05-2024 |
| SERVICE DEALER | AS EQUIPMENT PVT LTD - CU1A04116 (Rudrapur) | COMMISSIONING DATE |  |
| AMC STATUS | No AMC | WARRANTY STATUS | Out of Warranty |
| AMC REGISTRATION NO |  | LOAD HMR | 173 |
| AMC TYPE |  | UNLOAD HMR | 198 |
| AMC VISIT NO |  | TOTAL HMR | 372 |

| PARAMETERS | ACTUALS | RESPONSE |  | REPLACED DATE |
| --- | --- | --- | --- | --- |
| Is AF replaced | 1628 | No |  | 30-09-2025 |
| Is OF replaced | 1628 | No |  | 30-09-2025 |
| Is AOS replaced | 3628 | No |  | 30-09-2025 |
| Is Greasing done | 3628 | No |  | 30-09-2025 |
| Is Valve kit replaced | 7628 | No |  | 30-09-2025 |
| Is Pre filter replaced |  | NA |  |  |
| Is Fine filter replaced |  | NA |  |  |
| Is Carbon Filter replaced |  | NA |  |  |
| Is DSF Available |  | NA |  |  |
| Air End Model | X990637 | Yes |  |  |
| Air End Serial Number | BWGA105361 | Yes |  |  |
| Cooler Serial No |  | Yes |  |  |
| Fan Motor Make |  | Yes |  |  |
| Fan Motor Serial Number |  | Yes |  |  |
| Motor Make | Elgi | Yes |  |  |
| Motor Serial Number | MWGJ3M4222 | Yes |  |  |
| PLC Make | Neuron III | Yes |  |  |
| PLC Serial Number | A42449 | Yes |  |  |
| VFD Make |  | NA |  |  |
| VFD Serial Number |  | NA |  |  |
| Rated Pressure | 9.5 | Yes |  |  |
| FAD | 2.27m3/min | Yes |  |  |
| Oil used/ Is the Visual condition of Oil good | Air Lube XD/Good | Yes |  |  |
| Input Power (kW/HP) | 15Kw | Yes |  |  |
| Input Voltage (volts) | 400V | Yes |  |  |
| I Max Current | 35.6 Amp | Yes |  |  |
| Speed (RPM) | 1484 | Yes |  |  |
| Service Factor (SF) | 1.15 | Yes |  |  |
| Frame Size | 160 L | Yes |  |  |
| Customer power source (EB / GENSET) | Genset | Yes |  |  |
| Incoming Cable Type (CU - Copper /AL - Aluminium) | Cooper | Yes |  |  |
| Incoming Cable Size (sq.mm) | 16 Sq.mm | Yes |  |  |
| Isolator Make |  | Yes |  |  |
| Isolator type (HRC/ MCCB/ MPCB) | MCCB | Yes |  |  |
| Isolator rating (Amps) | 63 Amp | Yes |  |  |
| Backup fuse (Amps) | 63 Amp | Yes |  |  |
| Earthing cable size (sq.mm) | 16 Sq.mm | Yes |  |  |
| Earth leakage voltage less than 2V AC (Volts) | 0.6 | Yes |  |  |
| Starting method (DOL/YD/VFD/SD) | Star Delta | Yes |  |  |
| Motor insulation resistance value * (Mohms) |  | Yes |  |  |
| Control transformer primary tapping # (Volts) | 415V | Yes |  |  |
| Is the Transport locking clamps removed |  | Yes |  |  |
| Is the Separate earthing provided for Separator tank |  | Yes |  |  |
| Is the Separate earthing provided for Main motor |  | Yes |  |  |
| Is the Air end freely rotated by hand |  | Yes |  |  |
| Is the Oil filled as per our recommendation level |  | Yes |  |  |
| Is the Direction of rotation as per indicated on the main motor (Clockwise) |  | Yes |  |  |
| Is the AVM Condition good |  | Yes |  |  |
| Is the Sucker cooler fan good |  | NA |  |  |
| Is the Pusher cooler fan good |  | Yes |  |  |
| Load pressure (kg/cm2) | 8.0 | Yes |  |  |
| Unload pressure (kg/cm2) | 9.0 | Yes |  |  |
| High Sump Pressure Trip (kg/cm2) | 10.5 | Yes |  |  |
| High discharge oil temp. trip (°C) | 110°C | Yes |  |  |
| Dew point temp. (°C) |  | NA |  |  |
| Air filter change (hrs) | 2000 | Yes |  |  |
| Oil filter change (hrs) | 2000 | Yes |  |  |
| Air oil separator change (hrs) | 4000 | Yes |  |  |
| Oil change (hrs) | 4000 | Yes |  |  |
| Motor re-greasing (hrs) | 4000 | Yes |  |  |
| Main motor OLR setting (Amps) | 19 Amp | Yes |  |  |
| Fan motor OLR setting (Amps) | 1 Amp | Yes |  |  |
| VFD Max speed (rpm) |  | NA |  |  |
| VFD Min speed (rpm) |  | NA |  |  |
| VFD Optimum speed (rpm) |  | NA |  |  |
| Working pressure (kg/cm2) | 9.0 Bar | Yes |  |  |
| Sump pressure (kg/cm2) |  | Yes |  |  |
| Ambient temp. ** (°C) | 30 | Yes |  |  |
| Compressor room temp. ** (°C) | 34 | Yes |  |  |
| Discharge oil temp (°C) | 80 | Yes |  |  |
| Load hours (hrs) | 173 | Yes |  |  |
| Run hours (hrs) | 372 | Yes |  |  |
| Load current (Amps) - R | 30.4 | Yes |  |  |
| Load current (Amps) - Y | 29.6 | Yes |  |  |
| Load current (Amps) - B | 30 | Yes |  |  |
| Unload current (R/Y/B) (Amps) | 14 | Yes |  |  |
| Unload current (Amps) - Y | 14 | Yes |  |  |
| Unload current (Amps) - B | 15 | Yes |  |  |
| Voltage (Volts) - RY | 409 | Yes |  |  |
| Voltage (Volts) - YB | 407 | Yes |  |  |
| Voltage (Volts) - RB | 407 | Yes |  |  |
| Voltage drop during starting (Volts) | 08-10 Volt | Yes |  |  |
| Control mode (Auto/Local/DCS) | Local | Yes |  |  |
| VFD Speed (Load %) |  | NA |  |  |
| Is the Operation and Maintenance practice of the compressor and dryer, explained to the customer as per O&M manual |  | Yes |  |  |
| Is the OMM and Parts manual handed over to customer |  | Yes |  |  |
| Is the Uptime warranty explained to customer |  | Yes |  |  |
| Is the Uptime warranty documents signed by customer |  | Yes |  |  |
| Frequency (Hz) | 50 Hz | Yes |  |  |
| Fan motor current (Amps) | 0.8 | Yes |  |  |
| Frequency (Hz) |  | Yes |  |  |
| Fan Motor Current (amps) |  | Yes |  |  |
| Load count | 000052091 | Yes |  |  |
| Unload Sump Pressure (kg/cm 2) | 1.8 | Yes |  |  |
| NO OF VISITS MADE | 1 |  |  |  |
| PARTS RECOMMENDED FOR SERVICE |  |  |  |  |
| WORKDONE | Visited at the site for commissioning and found that customer self start the compressor.we checked machine machine incoming cable size found ok.we checked MCCB found 63 Amp.we checked earthing in compressor found ok.we checked electrically and mechanically connection found ok. |  |  |  |
| PARTS DUE FOR NEXT SERVICE |  |  |  |  |
| Oil Remaining Hours | 3,628 |  |  |  |
| Oil Replaced Date |  |  |  |  |
| Type of Oil |  |  |  |  |
| NEXT SERVICE HMR |  | NEXT SERVICE DUE ON |  |  |
| IS THE SERVICE CHARGES APPLICABLE | No | SERVICE CHARGES |  |  |
| ENGINEER | Hemant Kumar AE | CONTACT NO | 9773598925 |  |
| DEALER | AS EQUIPMENT PVT LTD - CU1A04116 (Rudrapur) |  |  |  |
| CUSTOMER FEEDBACK | satisfied |  |  |  |
| ENGINEER REMARKS | Customer installed the compressor on vibration pad then we informed to customer removed vibration pad and installation on levelled floor.informed to customer for changed Air Filter and Oil Filter. |  |  |  |

| NAME | SIGNATURE |
| --- | --- |